EXPANSION INTERFACE CARDS IN EXISTENCE

************************************************************

MEMORY EXPANSION INTERFACE CARDS

ADAM has a built-in RAM Memory of 64K. Many programs now require additional Expansion Memory, easily accomplished by inserting a 64K RAM Expansion Card into Slot #3 under the Top Access Cover of the ADAM Memory Console. To add larger memory up to 256K requires installation of the desired memory card in slot #3 AND a card that will address the memory above 64K in slot #2.  The Slot #2 Card can be a Printer Interface Card (# 119), dedicated Addressor card, or the Multi-purpose interface Card - MIB3 (# 468). A single wire connection is made between the two cards to complete the circuit for the extra memory recognition. All cards are designed with the latest technology for long life. Utility Software, such as TDOS and ADAM'S DESKTOP, can also utilize the larger Expansion Memory as a RAM DISK -- an electronic temporary drive that can save and load software data instantly when it is installed in the RAM DISK.  Programs like Adam's Desktop and others can make the ADAM a fast operating machine without addition of anything but an Expansion Memory Card.  Installs in right slot # 1

64K MEMORY CARD (Orphanware, Eve & others)

256K MEMORY CARD ((Orphanware, Eve & others)

512 MEMORY CARD (MI, E&T, & others)

1 MEG MEMORY CARD (MI, E&T)

2 MEG MEMORY CARD (Micro Innovations only)

PRINTER INTERFACE CARDS

Allows the use of a Parallel Dot Matrix Printer on the ADAM System. It can co-exist with the ADAM Daisy Wheel Printer, or become the exclusive printer of choice when a separate power supply is substituted for the ADAM Printer. A Dot Matrix Printer can print graphics of high quality, and depending on the type of dot matrix printer it can print in color or numerous letter quality fonts.  Some of the printer cards have a prom socket onboard to boot into a preprogrammed program. The Printer Card also serves to address memory on larger Memory Cards above 64K.   Installs in center slot #2

MIB2 INTERFACE CARD

Expansion card which allows the use of a Parallel Dot Matrix Printer as above.  It also

Has 2 serial ports onboard.  Some variations use an external power connection using the

ADAM’s input power cord.  Installs in center slot # 2

MIB3 INTERFACE CARD

Multi-purpose expansion card allows the same functions of the Printer Interface Card above, plus it provides Two Serial Ports that can be used to attach an External Modem (300 to 9600 Baud) and/or a Terminal (at 19,200 Baud) for display of 80 column text like found in CP/M and TDOS, or other serial devices.  It also has an onboard slot to install a boot prom if you have an IDE card in slot # 1.   Installs in center slot #2

DUAL SERIAL CARD

This interface card installs in left Slot #1 and provides the serial port functions as explained in the Multi-Purpose interface card description above.

ADAMLINK INTERNAL MODEM

300 Baud internal modem installs in Slot #1 under the Memory Console top access cover to provide telecommunication with other computers and Bulletin Board Systems. Uses .  ADAMLINK I , II, III, IV, V connection software. Installs in left slot #1

ADDRESSBOOK WITH AUTODIALER

Interface Card plugs into the right side expansion port of the ADAM Memory Console. Telephone line plugs into the interface so that AddressBook Software can dial numbers directly from your database. AddressBook also will print labels, etc.

Coleco Adam EVE VD-MB 80 Column Video/Expansion Unit 
and EVE SP-1 SS-CC Modules    Installs in right side connector
These are rare add-ons for your colecovision and/or adam computer

SP-1 is a Serial/Parallel Unit interface
SS-CC is a Speech Systhesizer/Clock Calender unit  
VD-MB/MON-80 is a true 80 Column Video Display Unit

MicroInnovations and Minnie Winnie hard drive interfaces for hard drives, zip drives, compact flash cards, and  Sparq drives

These cards are rare interface cards by 3rd party developers which allowed the ADAM

to access 20, 40 and 60 megabyte ide devices for storage and programming.

MicroFox Technologies (c. 2014) IDE cards, printer card w/ boot eprom, compact flash hard drives in 64mb and 100mb versions

These IDE cards area direct replacement for the MI cards.   The compact flash system

is a more reliable and faster hard drive idea.  It comes in a single 64mb version w/ 10 EOS partitions & 4 TDOS partitions OR a double 100mb version w/ 40 EOS partitions and 4 TDOS partitions.  Also a printer card w/ onboard eprom to boot  the IDE card. Available for the ADAM for the foreseeable future.

MI parallel port, serial port, boot prom interface

This card installs in the center slot #2 and serves a parallel printer card, serial card and also carries a socket for a boot eprom to allow the ADAM to boot into TDOS upon startup, or reset.

Addressor card (MI and Orphanware)

This card installs in center slot #2 and allows a memory expander larger than 64k to be recognized and used by the ADAM.