Coleco ADAM MIOCC pinouts

These were sent to me by Dr. Rich Drushel a while back.

Haven't been able to do anything with them yet, so I

thought I would share.

========================================================

MIOC Pinouts (U7):

/X = active low

Pin	Signal		I/O	Description

1	Vcc		input	+5V supply

2	RA7		output	8th bit DRAM refresh;

gated out by /ADDRBUFEN

3	BA15		input	Address BUS bit 15

4	BA14		input	Address BUS bit 14

5	BA13		input	Address BUS bit 13

6	/CVRST		input	ColecoVision reset switch

7	BD0		I/O	Data BUS bit 0

8	BD1		I/O	Data BUS bit 1

9	BD2 		I/O	Data BUS bit 2

10	BD3		I/O	Data BUS bit 3

11	/BWR'		input	made by complex logic from MA5,

/IORQ, and A10

12	BA6		input	Address BUS bit 7

13	BA7'		input	BA7, gated in by /ADDRBUFEN

14	/IORQ		input	IO Request - Z80 Control

15	/WAIT		input	Wait - Z80 control

16	/BUSAK		input	BUS Acknowledge - Z80 Control

17	/DMA		input	Asserted by Master 6801 to access

Z80 RAM

18	/BUSRQ		output	BUS Request - Z80 Control

19	/EOS_ENABLE	output	selects/deselects EOS ROM

20	/NET_RST	output	reset signal for ADAMNet

21	GND		---	Ground

22	/AUX_DECODE1	output	selects/deselects OS7 ROM

23	/RST		output	reset signal for the Master 6801

24	/CPRST		???	not connected on schematic;

reset for something

25	/PBRST		input	ADAM reset switch

26	/AUX_ROM_CS	output	selects/deselects expansion

ROM (center slot)

27	/ADDRBUFEN	output	enables/disables /BRD, /BWR,

/BRFSH,/BMREQ, /BM1, and /BIORQ

(disabled during DMA cycle)

28	/BOOT_ROM_CS	output	selects/deselects SmartWriter ROM

29	/245EN		output	enables/disables BD0-BD7

(disabled during DMA cycle)

30	/IS3		output	input to Master 6801

31	/OS3		input	output to Master 6801

32	/BMREQ		input	Memory Request - Z80 Control

33	/BRD		input	Z80 Control Signal

34	/BRFSH		input	Z80 Control Signal

35	/BM1		input	Z80 Machine Cycle 1

36	B(phi)		input	Z80 clock

37	MUX		output	multiplexing signal for

DRAM Refresh

38	/RAS1		output	row address strobe, base 64K RAM

39	/CAS1		output	column address strobe, base 64K RAM

40	/CAS2		output	column address strobe, 64K XRAM